Speedometer modelspoorbaan

Eddy Luursema, 18 februari 2022

Inleiding

In dit document wordt een eenvoudige snelheidsmeter voor de modelspoorbaan beschreven. Voor pakketten voor modelspoorbaanbesturing zoals WinDigipet en Traincontroller is een nauwkeurige ijking tussen ingestelde en daadwerkelijke snelheid noodzakelijk om een trein (locomotief) tot op de centimeter ergens te laten stoppen. Die pakketten rekenen in km/h voor de snelheid, maar in centimeters (in combinatie met de op te geven schaal) voor spoorlengtes.

De speedometer is gebaseerd op een eenvoudige Arduino Nano met twee infrarood lichtsluizen. Via meerdere kleine losse projecten wordt de uiteindelijke speedometer opgebouwd.

Materiaallijst

| Onderdeel | Aantal | Link | Prijs/stuk |
| --- | --- | --- | --- |
| Arduino Nano | 1 | https://www.tinytronics.nl/shop/nl/development-boards/microcontroller-boards/arduino-compatible/nano-v3.0-compatible | €6 |
| IR onderbrekingssensor | 2 | https://www.tinytronics.nl/shop/nl/sensoren/optisch/infrarood/infrarood-onderbrekingssensor | €3,50 |
| Drukknop | 2 | https://www.tinytronics.nl/shop/nl/schakelaars/manuele-schakelaars/drukknoppen-en-schakelaars/drukknop-module-rood | €1,25 |
| OLED scherm | 1 | https://www.tinytronics.nl/shop/nl/displays/oled/0.91-inch-oled-display-128*32-pixels-wit-i2c | €6,50 |
| Print | 1 | https://www.tinytronics.nl/shop/nl/gereedschap-en-montage/prototyping-toebehoren/experimenteer-printplaten/experimenteer-printplaat-7cm*9cm-dubbele-eilandjes | €0,65 |
| Schroefconnector 2 klemmen | 3 | https://www.tinytronics.nl/shop/nl/kabels-en-connectoren/connectoren/schroefterminals/2-pin-schroef-terminal-block-connector-5.08mm-afstand | €0,75 |
| Headers 30 pins | 1 | https://www.tinytronics.nl/shop/nl/kabels-en-connectoren/connectoren/pin-headers/female/40-pins-header-female | €0,50 |
| Draadjes |  |  |  |
| Adereindhulzen | 10 |  |  |
| Schroefjes/moertjes | 9 |  |  |
| Afstandsbussen | 5 |  |  |
| Multiplex plaat 40*8 cm | 1 |  |  |
| Plankjes 70*11 mm | 4 |  |  |
| Totaal |  |  | €24-30 |

Elektrisch schema

Constructie

Snelheid op modelspoorbaan berekenen

Gegeven dat een locomotief op een modelspoorbaan een bepaalde afstand L aflegt in T milliseconde betekent dat de locomotief rijdt met een snelheid van:

S = L / T mm/ms  (millimeter / milliseconde)

S = L / T / 1000  m/ms

S = L / T m/s

1 km/h = 1000 / 3600 m/s dus 1 m/s = 3.6 km/h

S = L / T * 3.6 km/h

Bij een schaal N met een verhouding 1:160 rijdt de locomotief dus in werkelijkheid een factor 160 sneller dus

S = L /T * 3.6 * 160 km/h

Testen  werking lichtsluizen

Attentie: in de documentatie van de leverancier https://www.tinytronics.nl/shop/nl/sensoren/optisch/infrarood/infrarood-onderbrekingssensor staat vermeld dat een pullup-weerstand van 1-10 kΩ nodig is; de interne pullup van een Arduino is volgens https://www.arduino.cc/en/Tutorial/Foundations/DigitalPins 20-50 kΩ. Daarom is een externe pullup -weerstand noodzakelijk.

/* Simple IR gateway detector, built in LED will respond to IR gateway */ const int PIN = 12; void setup() { pinMode(LED_BUILTIN, OUTPUT);; pinMode(PIN, INPUT_PULLUP); Serial.begin(9600); } void loop() { int value = digitalRead(PIN); digitalWrite(LED_BUILTIN,value); }

Dit programma is vooral handig om de werking te testen, maar ook als de sluis fysiek met plankjes gemonteerd moet worden om te controleren of de LED en detector recht tegenover elkaar staan. Verander de constante PIN afhankelijk van de te controleren sluis.

Meten snelheid van L naar R met twee sensoren

Bestand speedMeasurementFunction.ino is een library met een functie om de snelheid te berekenen. De twee elkaar opheffende stappen met mm en ms zijn commentaar geworden.

//const int MILLISPERSECOND = 1000; //const int MILLIMETERPERMETER = 1000; const float METERPERSECONDTOKMH = 3.6; float calculateSpeed(int sensorLength, long intervalMillis, int modelScale) { float speed = sensorLength / float(intervalMillis); // mm per msec //speed = speed / MILLIMETERPERMETER; // m per msec //speed = speed * MILLISPERSECOND; // m/s speed = speed * METERPERSECONDTOKMH; // km/h speed = speed * modelScale; // km/h in real scale return speed; }

/* Simple speed measuring system for model train control using two IR gateways Only suited for detection from L to R */ const int modelScale = 160; //the scale the model train is built in e.g. 1:160. Not a constant; can be changed later on const int sensorLength = 200; //the distance between the sensors in mm const int LEFTPIN = 12; const int RIGHTPIN = 11; long previousMillis; void setup() { pinMode(LEFTPIN, INPUT_PULLUP); pinMode(RIGHTPIN, INPUT_PULLUP); Serial.begin(9600); } void loop() { while (digitalRead(LEFTPIN) != 0) { //simple example wait for left sensor to be activated } previousMillis = millis(); while (digitalRead(RIGHTPIN) != 0) { //simple example wait for right sensor to be activated } long intervalMillis = millis() - previousMillis; float speed = calculateSpeed(sensorLength, intervalMillis, modelScale); Serial.print("Speed is "); Serial.print(speed); Serial.println(" km/h"); }

De snelheid is redelijk constant van deze dieselloc de BR204 van Fleischmann en ligt tussen de 121.93 en 122.30 km/h.

Met dit prototype kan prima worden gewerkt als van L->R wordt gereden en de schaal vast staat.

Lichtsluis tijdsmeting

Omdat het onbekend is of er door het onderbreken van de lichtstraal contactdender kan ontstaan net als bij een drukknop zijn met een eenvoudige logic analyzer de signalen van links en rechts bekeken en nagemeten of de tijd klopt met die in de software:

Links naar rechts op de voorzijde van de loc 1.653 ms zou betekenen

Op de achterzijde 1.650 ms:

Echter er brandden op de gebruikte baan een aantal rode seinen; deze geven zoals zal blijken helaas ook infrarode straling af. Deze seinen zijn nu uitgeschakeld; dit was mogelijk omdat de seindecoders zo zijn ingesteld dat deze geen seinbeelden geven, maar de individuele LED’s afzonderlijk bestuurbaar zijn.

De glitch links is verdwenen door het uitschakelen van het sein.

Aansturen OLED

Aangeschaft bij Tinytronics de https://www.tinytronics.nl/shop/nl/displays/oled/1.3-inch-oled-display-128*64-pixels-wit-i2c voor €8,- met als ingebouwde controller de SSH1106.

Uit: https://create.arduino.cc/projecthub/Arnov_Sharma_makes/0-96-inch-oled-getting-started-guide-78163a. Helaas bestaan er ontzettend veel verschillende OLED- en LCD-controllers, die in veel gevallen uitgerust zijn met de SSD1306. Er moet dus ook een library toegepast worden en die van de SSH1106 en SSD1306 zijn niet compatibel. Op de website van Tinytronics staat een verwijzing naar een SSH1106-library https://github.com/olikraus/U8g2_Arduino. Op zich werkt deze library, maar die heeft ontzettend veel grafische componenten waardoor na compilatie bijna 24k programmageheugen is verbruikt. Daarom is gekeken naar een kleinere library: https://github.com/notisrac/SH1106Lib

Op schaalgrootte 2 moeten wat concessies worden gedaan t.a.v. het aantal tekens op één regel:

/********* Rui Santos Complete project details at https://randomnerdtutorials.com Adapted for train by Eddy *********/ #include <Wire.h> #include <Adafruit_GFX.h> #include <Adafruit_SSD1306.h> #define SCREEN_WIDTH 128 // OLED display width, in pixels #define SCREEN_HEIGHT 32 // OLED display height, in pixels // Declaration for an SSD1306 display connected to I2C (SDA, SCL pins) Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1); void setup() { Serial.begin(115200); if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) { // Address 0x3D for 128x64 Serial.println(F("SSD1306 allocation failed")); for (;;); } delay(2000); display.clearDisplay(); display.setTextSize(2); //size 2 does work with less letters display.setTextColor(WHITE); display.setCursor(0, 0); display.println("N 123.4kmh"); display.setCursor(0, 17); display.println("432.1 #999"); display.display(); } void loop() { }

.

Omschakelen schaal

Het bestand controlScale_Functions.ino bevat library voor omzetten en printen van de aschaal

// Function to set new scale given old one. There no such thing as getting the next in an enum int setNextScale(int oldScaleSetting) { int returnValue; switch (oldScaleSetting) { case G: returnValue = O; break; case O: returnValue = S; break; case S: returnValue = H0; break; case H0: returnValue = N; break; case N: returnValue = Z; break; case Z: returnValue = G; break; } return returnValue; } // Function to print the scale. There is no easy way to print enum contents void printScale(int currentScale) { String scaleLetter; switch (currentScale) { case G: scaleLetter = "G"; break; case O: scaleLetter = "O"; break; case S: scaleLetter = "S"; break; case H0: scaleLetter = "H0"; break; case N: scaleLetter = "N"; break; case Z: scaleLetter = "Z"; break; } Serial.print(scaleLetter); }

/* Simple control example setting the scale of the system using a button */ enum SCALE {G = 24, O = 48, S = 64, H0 = 87, N = 160, Z = 220}; SCALE scaleSetting; //actual scale set const int CONTROLPIN = 11; void setup() { pinMode(CONTROLPIN, INPUT_PULLUP); scaleSetting = N; Serial.begin(9600); } void loop() { while (digitalRead(CONTROLPIN) != 1) { //simple example wait for button to be activated }; scaleSetting = setNextScale(scaleSetting); delay(10); while (digitalRead(CONTROLPIN) != 0) { //simple example wait for button to be deactivated }; delay(10); Serial.print("scale: "); printScale(scaleSetting); Serial.println(); }

Project 4 OLED schermbesturing

Speedometer

speedCalculationFunction.ino

/* Calculates speed of loc given the time interval passing two IR gateways according to scale and length of gateway */ //const float MILLISPERSECOND = 1000.0; //const float MILLIMETERPERMETER = 1000.0; const float METERPERSECONDTOKMH = 3.6; float calculateSpeed(float sensorLength, long intervalMillis, int modelScale) { float speed = sensorLength / float(intervalMillis); // mm per msec //speed = speed / MILLIMETERPERMETER; // m per msec //speed = speed * MILLISPERSECOND; // m/s speed = speed * METERPERSECONDTOKMH; // km/h speed = speed * float(modelScale); // km/h in real scale return speed; }

irGateWay.ino

/* All the setup and functions needed to use the two interrupt pins to detect the passing of the IR gateway */ const int LEFTPIN = 2; //interrupt capable pin must be 2 or 3 on a Arduino Uno/Nano const int RIGHTPIN = 3; //interrupt capable pin must be 2 or 3 on a Arduino Uno/Nano void setupIRGateWay () { pinMode(LEFTPIN, INPUT_PULLUP); pinMode(RIGHTPIN, INPUT_PULLUP); attachInterrupt(digitalPinToInterrupt(LEFTPIN), interruptLeft, FALLING); //IR gateway goes to LOW when IR is blocked attachInterrupt(digitalPinToInterrupt(RIGHTPIN), interruptRight, FALLING); } void interruptLeft() { currentMillis = millis(); //directly after interrupt to assure accurate measurement if (passState == NONE) { //if there would be second interrupt due to bouncing effect of IR gateway, no problem. passState = LEFTIN; //loc is coming from left, save that printDirection = true; //signal main program loop to print direction only once previousMillis = currentMillis; //time of entrance is saved } else if (passState == RIGHTIN) { //loc came from right and is now leaving left, start calculation intervalMillis = currentMillis - previousMillis; //intervalmillis <> 0 means main program loop must print it passState = NONE; } } void interruptRight() { currentMillis = millis(); //directly after interrupt to assure accurate measurement if (passState == NONE) { //if there would be second interrupt due to bouncing effect of IR gateway, no problem. passState = RIGHTIN; //loc is coming from left, save that printDirection = true; //signal main program loop to print direction only once previousMillis = currentMillis; //time of entrance is saved } else if (passState == LEFTIN) { //loc came from left and is now leaving right, start calculation intervalMillis = currentMillis - previousMillis; //intervalmillis <> 0 means main program loop must print it passState = NONE; } }

controlScale_Functions

/* Functions to alter and to print the scale of the speedometer */ SCALETYPE setNextScale(SCALETYPE oldScaleSetting) { //given current scale, return next scale. SCALETYPE returnValue; switch (oldScaleSetting) { case G: returnValue = O; //Attention sequence in case statement must match enum sequence in main program break; case O: returnValue = S; break; case S: returnValue = H0; break; case H0: returnValue = N; break; case N: returnValue = Z; break; case Z: returnValue = G; break; } return returnValue; } String printScale(SCALETYPE currentScale) { String scaleLetter; switch (currentScale) { case G: scaleLetter = "G"; break; case O: scaleLetter = "O"; break; case S: scaleLetter = "S"; break; case H0: scaleLetter = "H0"; break; case N: scaleLetter = "N"; break; case Z: scaleLetter = "Z"; break; } return scaleLetter; }

OLED

/* All the setup and functions needed to use the OLED screen Print is scalable in several sizes. With size 2 compromises are made in number of characters shown */ #include <Wire.h> #include <Adafruit_GFX.h> #include <Adafruit_SSD1306.h> #define SCREEN_WIDTH 128 // OLED display width, in pixels #define SCREEN_HEIGHT 32 // OLED display height, in pixels // Declaration for an SSD1306 display connected to I2C (SDA, SCL pins) Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1); void setupOLED () { if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) { // Address 0x3D for 128x64 Serial.println(F("SSD1306 allocation failed")); for (;;); } display.clearDisplay(); display.setTextSize(1); display.setTextColor(WHITE); display.setCursor(0, 0); display.println("Speedometer V0.1"); //print program, version and author display.setCursor(0, 17); display.println("Eddy Luursema"); display.display(); delay(4000); } void printOutput(int scaleSetting, float currentSpeed, float averageSum, int averageCount, state passState) { String outputOLED; display.clearDisplay(); display.setTextSize(2); display.setTextColor(WHITE); display.setCursor(0, 0); outputOLED = outputOLED + printScale(scaleSetting) + " "; if (passState != NONE) { if (passState == LEFTIN) { outputOLED = outputOLED + ">> "; } else { outputOLED = outputOLED + "<< "; } } else { outputOLED = outputOLED + String(currentSpeed, 1); if (outputOLED.length() > 6) { //string does not fit with km/h so print kmh outputOLED = outputOLED + "kmh"; } else outputOLED = outputOLED + "km/h"; } display.print(outputOLED); display.setCursor(0, 17); if (averageCount >= 1) { //average can be calculated display.print(averageSum / averageCount); display.print(" "); display.print("#"); display.print(averageCount); } display.display(); }

Buttons.ino

/* All the setup and functions needed to use the two buttons with deboucing. Software is capable of handling more buttons */ int buttonPins[NRBUTTONS] = {A0, A1}; //pins to which buttons are connected long buttonActionMillis[NRBUTTONS]; //time first action on given button const int DEBOUNCE = 10; //10 msec for debouncing void setupButtons() { for (int button = 0; button < NRBUTTONS; button++) { pinMode(buttonPins[button], INPUT_PULLUP); //set pin to input buttonStates[button] = OPEN; //button state is default not pressed } } void testButtons() { for (int button = 0; button < NRBUTTONS; button++) { //wal;k through all buttons if ((buttonStates[button] == OPEN) && (digitalRead(buttonPins[button]) == 0)) { //was button pressed? buttonStates[button] = ACTION; //this state is used as semaphore between this function and main loop buttonActionMillis[button] = millis(); //save moment of first press } else if ((buttonStates[button] == TOCLOSE) && (millis() - buttonActionMillis[button] > DEBOUNCE)) { //button was closing and debounce time has passed? buttonStates[button] = CLOSED; //button is now officially closed } else if ((buttonStates[button] == CLOSED) && //button is opening? (digitalRead(buttonPins[button]) == 1)) { //button pressed first time buttonStates[button] = TOOPEN; buttonActionMillis[button] = millis(); } else if ((buttonStates[button] == TOOPEN) && (millis() - buttonActionMillis[button] > DEBOUNCE)) { //button was opening and debounce time has passed? buttonStates[button] = OPEN; //button is now officially opened } } } BUTTONSTATE getButtonState(BUTTONTYPE button) { return buttonStates[button]; }

/* Speedometer by Eddy Luursema 2022 Febrary Simple speed measuring system for model train control using two IR gatesways, two buttons and an OLED This example uses interrupts on pin2 and 3 of the Atmel386P which can be configured for interrupts for a locomotive coming from left of right */ enum SCALETYPE {G = 24, O = 48, S = 64, H0 = 87, N = 160, Z = 220}; //these are the scales system supports, adding a new one means also change controlScaleFunctions.ino SCALETYPE scaleSetting; //actual scale that is set const int NRBUTTONS = 2; //system has two buttons int buttonStates[NRBUTTONS]; //buttonstate is used to prevent bouncing but also as semaphore enum BUTTONTYPE {SCALEBUTTON, RESETBUTTON}; //name of the buttons in this program enum BUTTONSTATE {OPEN, ACTION, TOCLOSE, CLOSED, TOOPEN}; //button can be open, closed, bouncing from open to close or open and action const float sensorLength = 348.0; //the distance between the sensors in mm specific for each construction enum state {NONE, LEFTIN, RIGHTIN}; //possible statuses of the loc being outside the gateway, entered from left or entered from right volatile state passState; //catual status of the loc; is volatile because is used as semaphore const int GLITCH = 30000; //the loc must pass in 30 seconds or passing of IR will be treated as a glitch and loc is supposed to be outside long previousMillis; //timestamp of previous passing long currentMillis; //timestamp of previous passing long intervalMillis; //interval of passing two IR-gateways float currentSpeed; //calculated speed according to scale float averageSpeed; //average speed over multiple measurements; button can reset average int averageCount; //number of measurements volatile bool printDirection; //indication that main program must print direction; is volatile because is used as semaphore void setup() { setupButtons(); //initialise input ports setupOLED(); //initialise OLED; show hello for a few seconds setupIRGateWay(); //initialise IR input ports and interrupts scaleSetting = N; //my track is scale N so that is the default passState = NONE; //start without locomotive between gateways printDirection = false; //used as semaphore between interrupt routine and main loop. If true print direction intervalMillis = 0; //used as semaphore between interrupt routine and main loop. If unequal 0 print interval and speed averageSpeed = 0.0; averageCount = 0; printOutput(scaleSetting, 0, averageSpeed, averageCount, passState); //print scale and average zero } void loop() { testButtons(); //test if any button is pressed if (buttonStates[SCALEBUTTON] == ACTION) { //scale button pressed? scaleSetting = setNextScale(scaleSetting); //set the next scale and print it printOutput(scaleSetting, currentSpeed, averageSpeed, averageCount, NONE); buttonStates[SCALEBUTTON] = TOCLOSE; //semaphore: button pressed was detected by function testButtons. Main loop handles action and set state to TOCLOSE so function testButtons can catch debounce } if (buttonStates[RESETBUTTON] == ACTION) { //reset button pressed? averageSpeed = 0; //reset average and print it averageCount = 0; currentSpeed = 0; printOutput(scaleSetting, currentSpeed, averageSpeed, averageCount, passState); buttonStates[RESETBUTTON] = TOCLOSE; //reset semaphore } if (printDirection) { //semaphore indicates one of the interrupt functions has direction to print printOutput(scaleSetting, currentSpeed, averageSpeed, averageCount, passState); printDirection = false; //reset semaphore so direction is only printed once } if (intervalMillis != 0) { //semaphore indicates one of the interrupt functions has interval and speed to print currentSpeed = calculateSpeed(sensorLength, intervalMillis, scaleSetting); averageSpeed = averageSpeed + currentSpeed; averageCount++; printOutput(scaleSetting, currentSpeed, averageSpeed, averageCount, NONE); intervalMillis = 0; //reset semaphore so interval is only printed once } if ((passState != NONE) && (millis() - previousMillis > GLITCH)) { //both IR gatways not within 30 seconds? passState = NONE; //glitch in IR so assume loc is outside of gateway printOutput(scaleSetting, currentSpeed, averageSpeed, averageCount, NONE); } }